1/15/2008


ANSWER KEY – ASA-100 TEST
1. Section 1D:  The distributor shall notify the accreditation organization, in writing, of any significant changes to its quality system and receive written notification of the acceptance of the change prior to implementation.

2.
Section 1E: 3 of 11 possible answers (please review section 1E)
(i) 
the organization's training requirements and records,


(ii) 
receiving inspection procedures,

(iii) the parts identification system employed

      3.  
Section 2A:  [For compliance with ASA-100] Self-audit/evaluations shall be conducted, at minimum, on an annual basis.
4. Section 2A1:  

(i)
corrective action shall be appropriate and prompt,

(ii)
correct the discrepancies reported,
(iii) locate and correct similar discrepancies, if they exist, in areas not audited,

(iv) correct the root cause of the problem evidenced by the discrepancies,
(v) implement follow-up action(s)to assure no recurrence
5. Section 3D:  The distributor shall have a system in place to segregate and identify serviceable from unserviceable parts in a manner that will control the issuance of those parts.
Section 8B: Batch segregation shall be maintained for parts so identified by the manufacturer, such as aircraft fasteners.  

Section 8H: Rejected materials shall be identified as such and segregated from usable stock.

Section 8H1:  Aircraft parts, and parts that could be reasonably assumed to be sold for aircraft use, shall be segregated from non-aircraft parts.

6.
Section 4C:  All training, both formal (classroom) and on-the-job training (OJT), shall be documented and the records shall be maintained for all employees who underwent training.  
7.  Section 4D:  The distributor shall maintain a roster of the personnel and their
alternates authorized to perform inspection functions and identify the inspection   function(s) that each persons authorized to perform.

        8.  Section 5A:  Procurement; Production; a FAA certificate holder.

        9.  Section 5C:  The distributor shall maintain a list of their approved suppliers and

             a quality history for each source.

10. Section 6A:  3 of 5 possible answers (please review Section 6A)


  (i)
  check for any obvious physical damage,

  (ii)
  verification that all appropriate plugs and caps are installed, if applicable,

(iii) verification that part numbers, model numbers, serial numbers, lot and/or

batch numbers, etc., of the items, match the accompanying documentation.
        11. Definition Section: LIs are used to provide guidance regarding ASA-100 and/or

  FAA    AC 00-56A.  LIs are required if applicable to your business.  Each LI

  addresses requirements and future expectation.  

        12. Section 6D:  Inspection stamp identification imprints shall not be reused for two

              years after an inspector to whom the imprint was assigned leaves the position. 

        13. Section 7B:  The distributor shall have procedures to prevent tools/equipment

              which are past due calibration from being used.  Each unit in the calibration

              program shall be traceable to the standard against which it was calibrated.

             Current documentation of calibration status shall be maintained.

        14. Section 8B:  Batch segregation shall be maintained for parts so identified by
   the manufacturer, such as aircraft fasteners.  The system shall include 
   procedures for splitting of lots and the documentation of such splitting.
   Purchases, less sales, should equal inventory, which shall balance on batch/lot
   numbered inventories.
         15. Section 8H:  A closed loop system shall exist to implement corrective action
                following the detection of substandard or otherwise non-conforming parts.
                Rejected materials shall be identified as such and segregated from usable
                stock.
          16. Section 8I1:  Records and documents shall be maintained on all serialized

    scrapped parts.  The procedure shall identify by title or position the individual

    responsible for verifying that parts were adequately mutilated before discard.

               Section 8I2:  The distributor shall maintain a record of all life-limited parts                        scrapped out. The record shall contain a description of the part, its part                number and serial number, if applicable, and the date the part was scrapped.                 The dealer shall retain this record for at least seven years.

               Section 8I3: The distributor shall impose these same requirements on their                subcontractors and/or repair facilities with whom they do business.

           17. Section 9A:  Out-of-time or outdated parts and materials shall either be

    segregated from serviceable items, or discarded. The program shall specify a

    system that will assure that no expired material or part will be issued. This 

    program includes component subassemblies containing shelf life-limited parts.

          18. Appendix A, Documentation Matrix: Approval for return to service attached to                     part, product, or appliance.
          19. Section 10D:  the source of production or to an FAA certificate holder
    20. Section 11A: The distributor quality system shall require components and parts
             to be shipped in an ATA-300 Specification container or equivalent as              appropriate for the unit being shipped, or as specified by the customer.  The           item should be packed in the container in a manner that will preclude damage           from rough handling of the container.

    21.  Section 11B7:  Verification that all appropriate required documentation are at            hand, properly completed, and signed.

          22.  Section 12A:  The distributor shall maintain documentation of traceability for

                 at least 7 years from the date of sale to the customer. Documents shall

                 demonstrate serial number, or lot & batch traceability, when applicable. The

                 distributor shall maintain a filing system such that the data is readily available

                 and identifiable for each customer, each purchase.

23. Section 8I2:  The distributor shall maintain a record of all life-limited parts          scrapped out. Records shall contain a description of the part, its part number and serial number, if applicable, and the date the part was scrapped. The dealer shall retain this record for at least seven years.

     Section 12A: The distributor shall maintain documentation of traceability for at       least 7 years from the date of sale to the customer.

                 Section 12C: Records confirming fastener integrity, including physical and                  chemical test reports, shall be maintained for a minimum of seven years.
24.  Section 13A:  Technical data, when required, shall be maintained in a manner     that ensures such data is up-to-date and accessible as appropriate. Hand  entries or corrections to technical data are not acceptable.

25.  Shipping

PAGE  
1
rev. 3.4

©Aviation Suppliers Association

